XIRCOM TOKEN RING ADAPTER

Information Not Yet Updated

Additional Information

SmartRing Software Version 4.0

October 29, 1993

======================================================================

Welcome to a new level of connectivity convenience!  The Xircom Token

Ring Adapters are the easiest-to-use, most compatible parallel port

Token-Ring adapters available.

Please refer to your network operating system manual and your Xircom

User's Guide for instructions on installation of the drivers contained

on this diskette.  This .DOC file contains information that may not

be in your User's Guide.

GENERAL

=======

This release of SmartRing(tm) software supports all Xircom Token Ring

Adapters:  Pocket Token Ring Adapter, External Token Ring Adapter,

Pocket Token Ring Adapter II, and the new Pocket Token Ring Adapter III.

It does not support Xircom's first 4Mbps-only adapter, models PT04B3

and PT04BT.  If you require updated software for your PT04B3 or PT04BT,

please contact Xircom Technical Support for an adapter upgrade.

ENHANCEMENTS

============

The enhancements and features of this release are:

DOS drivers:

1)   Xircom Parallel Port Multiplexor (PPX) support has been added

to all DOS drivers

2)   The DOS ODI driver (TRODI.COM) now supports promiscuous mode for

use with network analyzer products such as Novell's LANalyzer

3)   SMARTIPX now supports Microsoft Windows v3.1

4)   NetWare 4.0 and 4.01 are supported with TRODI.COM

5)   486 - 66mhz machines, as well as IBM ThinkPads at 50mhz are now

supported

6)   NEC Versas now work in EPP mode

7)   EPP drivers work on both Intel 386SL and 486SL-based machines,

and support EPP BIOS functionality (where EPP is supported

directly by the machine's BIOS and does not require a separate

EPP.COM or EPP.SYS file to be loaded)

OS/2 drivers:

1)   Enhanced Parallel Port (EPP) capabilities for all OS/2 drivers

on both Intel 386SL and 486SL-based machines

2)   A NetWare Requester for OS/2 (TRODI.OS2) has been added

3)   SMARTND.OS2 now supports OS/2 2.1

Other:

TRTEST /x will return a DOS errorlevel based on the condition of the

adapter.  This errorlevel may be used in a batch file to determine if

the adapter is functioning before attempting to load a driver.  The

errorlevel returned is:

0  The adapter is responding

1  A Xircom driver is already loaded

2  A Xircom adapter could not be found

LAN SUPPORT PROGRAM (SMARTLSP) NOTES

====================================

When using the External Token Ring Adapter (ET16BU) and SMARTLSP

with DOS LAN Requester, an intermittent loss of network connection

may occur.  Please contact Technical Support if you experience

this situation.

NDIS NOTES

==========

This version of SMARTND.DOS does not support CAF (capture all

frames) mode for products such as FTP's LANwatch.

ODI NOTES

=========

If you are using a NET.CFG configuration file with your ODI driver,

using "=" is not a valid syntax.

For example, use INT 7 instead of INT=7.

Also, manual selection of the LPT value is not supported.

PPX NOTES

=========

Software support for the Xircom Parallel Port Multiplexor (PPX) is

programmed into the DOS drivers provided on this diskette.  The

network driver will detect whether or not PPX hardware is present

when being used with a Pocket Token Ring Adapter.  DO NOT load a

separate PPX driver (i.e., PPX.COM or PPX.SYS).

When you use the Xircom adapter with a Xircom Parallel Port Multiplexor

(PPX) and a printer, be sure that your printer is turned on before

you load your network drivers.

The SMARTND.OS2 and TRODI.OS2 drivers do not support the Xircom Parallel

Port Multiplexor (PPX).  Local printing is supported under DOS only.

Some machines with Enhanced Parallel Port (EPP) capability may not

support EPP mode when used in conjunction with a PPX.

Local printing is not working properly with the driver SMARTND.DOS

under Windows.

OS/2 NOTES

==========

Local (pass-through) printing with the External Token Ring Adapter,

or with a Pocket Token Ring Adapter and PPX is not supported under OS/2.

MACHINE SPECIFICS

=================

The drivers in this release support machines with Enhanced Parallel

Port (EPP) capabilities.  For additional information on EPP, refer

to the User's Guide.

To enable the EPP utility on the COMPAQ LTE Lite, first use Compaq's

SETUP program to change the default LPT port to LPT2.

The NEC UltraLite Versa with ROM BIOS v1.01.06 supports EPP mode in

the BIOS (you do not need to use a separate EPP.COM or EPP.SYS file).

However, if a Xircom driver is loaded, unloaded (for example, typing

TRODI /u), then reloaded a second time, or if TRTEST or TRSETUP are

run more than once, a "Xircom adapter missing, not powered, or mal-

functioned" error message will be displayed.  To prevent this

occurrence, simply run the Xircom EPP.COM utility prior to loading

the driver as follows:

EPP Enb

TRODI   (or TRTEST, or TRSETUP)

When using a Mitsuba Ninja or Packard Bell 286 LPV under NetWare,

Interrupt 7A must be disabled by creating a SHELL.CFG file containing

the line:  INT7A=OFF

XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call or send

a facsimile to Dave Green, as directed in the Readme.  Access the Xircom

INFOSYS BBS at up to 14400 bps, No parity, 8 data bits, 1 stop bit

(N-8-1).

-------------------------------------------------------------------------

Some of the product names herein have been used for identification

purposes only and may be trademarks of their respective companies.

===============================[  END ]==================================